| Location / Sites: Various | Task / Activity: Working in Confined Spaces (Existing Manholes and Deep Excavations) | Personnel: All |
| --- | --- | --- |

| Name: Matt Traynor | Position: Managing Director | Date: Jan 2022 | Ref No: CS01 |
| --- | --- | --- | --- |

| No | Hazard Identification | Associated risks | Who & Number at risk | Existing control measures | Consequence | likelihood | Current risk rating | Additional controls | Consequence | likelihood | Post risk rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Restricted access/egress | Trips, slips, falls | All | Eye protection/PPE Approved footwear Ladders/platforms safely secured Harnesses/personal lines | 4 | 3 | 12 | Ladders/platforms safely secured Excavations/Platforms fitted with guard rails Harnesses/personal lines Working at height regulations 2005 | 4 | 2 | 8 |
| 2 | Contamination residue | slips, skin/tissue damage, disease (leptospirosis) | All | Eye protection/PPE Approved footwear Approved and certified equipment | 3 | 3 | 9 | Eye protection/PPE/Approved footwear Approved and certified equipment Barrier cream/washing facilities COSHH Regulations 2002 Environmental protection act 1990 part III Control of pollution act 1974 part III sections 60, 61, 71 Control of pollution (oil storage) (England) regulations 2001 Waste management : The duty of care: A code of practice 1996 | 3 | 2 | 6 |
| 3 | Oxygen deficiency | Asphyxiation | All | Breathing apparatus PPE | 5 | 2 | 10 | Operator minder monitoring harnesses/personal lines | 5 | 1 | 5 |
| 4 | Hydrocarbon vapour | Asphyxiation | All | Breathing apparatus PPE | 5 | 2 | 10 | Operator minder monitoring harnesses/personal lines | 5 | 1 | 5 |
| 5 | Fire and/or explosion | Loss of life/loss of property/major accidents | All | No smoking policy No naked flames Approved footwear and PPE | 5 | 3 | 15 | No smoking policy No naked flames Approved footwear and PPE Planning (control of major hazards) regulations 1999 Fire precautions (workplace) regulations 1997/1999 Fire safety order (October 2006) Construction fire safety CIS 51 Fire and explosion – How safe is your workplace INDG370 issue 4/04 | 5 | 2 | 10 |
| 6 | Noise | Hearing damage | All | Ear defenders provided | Noise assessments to be carried out inside confined space. The first action level is 80dB(A) LEPd when workers may request hearing protection which must be provided. At the second action level of 85dB(A) LEPd hearing protection must be provided and must be worn. All rail personnel are provided with hearing protection as part of their PPE issue. Every attempt will be made by to reduce noise levels at source. |  |  |  |  |  |  |

Matt Traynor

Managing Director

January 2022