| Location / Sites: Various | Task / Activity: # Generic Construction Site Management | Personnel: All |
| --- | --- | --- |

| Name: Matt Traynor | Position: Managing Director | Date: Jan 2021 | Ref No: ConGEN01 |
| --- | --- | --- | --- |

| No | Hazard Identification | Associated risks | Who & Number at risk | Existing control measures | Consequence | likelihood | Current risk rating | Additional controls | Consequence | likelihood | Post risk rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Fall of tools/equipment when working aloft | Damage to tools/equipment, injury to workers/public | All | Induction, working at heights training | 4 | 2 | 8 | Hardhat areas below, competence and familiarity with environment Tool box talks, assessment of competency. Catch netting. Cordon areas adjacent to public. | 4 | 1 | 4 |
| 2 | Materials/equipment hit by moving site vehicles | Damage to site vehicles, damage to tools/equipment, injury | All | Induction, competence and familiarity with environment Designated roadways, walk ways, storage points | 4 | 2 | 8 | Work to Method statement and site specific risk assessments and adhere to roadways, walk ways, storage points. Appoint trained and competent banksman to control personnel/traffic. | 4 | 1 | 4 |
| 3 | Worker hit by moving site vehicles | Injury to worker | All | Induction, competence and familiarity with environment Designated roadways, walk ways, storage points | 4 | 2 | 8 | Work to Method statement and site specific risk assessments and adhere to roadways, walk ways, storage points. Appoint trained and competent banksman to control personnel/traffic. | 4 | 1 | 4 |
| 4 | Fumes from plant/vehicles | Smarting eyes, coughing, bronchial irritation | All | Properly maintained plant/vehicles and equipment | 3 | 3 | 9 | Careful placement of static plant Avoidance of areas and use of PPE e.g. respirators where required and adhere to roadways, walk ways, storage points | 3 | 2 | 6 |

| 5 | Use of small plant | Flying debris striking worker causing major injury Fire causing loss, damage, major injury | All | Process likely to produce flying debris not to be undertaken in unprotected areas. Area surveyed for possible fire hazards prior to work commencing | 4 | 2 | 8 | Only staff with proficiency certificates and subject to monitoring to be used and such work to be supervised Appropriate fire extinguisher to be located at site of work. | 4 | 1 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Poor housekeeping | Increased risk of incident and damage to equipment/plant slips trips and falls | All | Suitable level of supervision Allocation of responsibility of specific area to individuals | 3 | 3 | 9 | Ad hoc and planned monitoring Suitable designated storage and disposal posts. Toolbox talks keep the site tidy | 3 | 2 | 6 |
| 7 | Storage of materials/plant | Struck by site vehicle Damage to site vehicles, damage to tools/equipment, injury | All | Designated and sufficient storage areas Designated roadways and walk ways | 4 | 2 | 8 | Briefing to use the designation of plant mounting equipment secure storage areas keep site tidy regime | 4 | 1 | 4 |
| 8 | Site vehicles on/near the roadside | Struck by road vehicles injury or loss of life. Damage to tools, vehicles | All | Road vehicle access to be in accordance with site access arrangements | 4 | 2 | 8 | Site vehicles only operate only within the areas designated. Use of competent banksmen to control site vehicle movements. | 4 | 1 | 4 |
| 9 | Incursion of tools & equipment on to site roadways/walkways | Damage to site vehicles and equipment, injury | All | Induction and vigilance by workers and supervision, | 4 | 3 | 12 | Safe system of work and compliance with the competence training and recognition of the need to work safely. | 4 | 2 | 8 |
| 10 | Spillage of diesel oil | Risk of Fire. Risk of slip Contamination | All | Diesel oil to be stored in purpose designed container. COSHH training and spill kit instruction | 3 | 2 | 6 | Only enough diesel oil to be taken to site for work required. Diesel oil container to be anchored in an upright position when travelling and in use Awareness to avoid contamination, welfare facilities and sufficient PPE | 3 | 1 | 3 |

| 11 | Spillage of petrol | Risk of Fire. Risk of slip Contamination | All | Petrol to be stored in purpose designed container. COSHH training and spill kit instruction | 3 | 2 | 6 | Only enough petrol to be taken to site for work required. Petrol container to be anchored in an upright position when travelling and in use Awareness to avoid contamination, welfare facilities and sufficient PPE | 3 | 1 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | Spillage of lubricating oil | Risk of Fire. Risk of slip Contamination | All | Lubricating oil to be stored in purpose designed container. COSHH training and spill kit instruction | 3 | 2 | 6 | Only enough lubricant to be taken to site for work required. Lubricant container to be anchored in an upright position when travelling and in use Awareness to avoid contamination, welfare facilities and sufficient PPE | 3 | 1 | 3 |
| 13 | Spillage of hydraulic fluid | Risk of fire. Risk of slip, Contamination | All | Hydraulic oil to be stored in purpose designed container. COSHH training and spill kit instruction | 3 | 2 | 6 | Only enough hydraulic fluid to be taken to site for work required. Hydraulic fluid container to be anchored in an upright position when travelling and in use Awareness to avoid contamination, welfare facilities and sufficient PPE | 3 | 1 | 6 |
| 14 | Poor/change in environmental conditions | Flood landslip Increased risk of fatality, major injury and damage | All | Avoidance of work in exposed areas during times of inclement weather through planning process | 4 | 2 | 8 | Emergency plan to contain inclement weather procedure. Effective serious and imminent danger procedures briefed in and understood | 4 | 1 | 4 |
| 15 | Production of excessive noise | Hearing loss distraction, lack of communication | All | Safe system of work ear defenders | The client companies normally carry out noise assessments. The first action level is 80dB(A) LEPd when workers may request hearing protection which must be provided. At the second action level of 85dB(A) LEPd hearing protection must be provided and must be worn. All rail personnel are provided with hearing protection as part of their PPE issue. Every attempt will be made by NCS to reduce noise levels at source. |  |  |  |  |  |  |
| 16 | Sudden noise | Nervous reaction | All | Safe system of work | 3 | 2 | 6 | Brief of area worked in, removal from area and PPE | 3 | 1 | 6 |
| 17 | Waste not removed from site | Vandalism. Items placed across roadways/walkways. Waste used as a projectile/weapon resulting in fatality, major injury, injury | All | All responsible for inspection of site for poor housekeeping | 4 | 2 | 8 | Re-cycling stations to be established with designated containers/skips. Waste to be removed by approved waste disposal agent. | 4 | 1 | 4 |

Matt Traynor

Managing Director

January 2021