| Location / Sites: Various | Task / Activity: Rail Infrastructure Management - Handback Engineer | Personnel: All |
| --- | --- | --- |

| Name: Matt Traynor | Position: Managing Director | Date: Jan 2021 | Ref No: RSM09 |
| --- | --- | --- | --- |

| No | Hazard Identification | Associated risks | Who & Number at risk | Existing control measures | Consequence | likelihood | Current risk rating | Additional controls | Consequence | likelihood | Post risk rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Insufficient ballast shoulder/poor ballast distribution | Movement of track: Increasing wear Affecting clearances Causing cyclic top/uneven ride/twist faults Formation of wet beds Temporary speed restrictions Emergency speed restrictions De-railment | All | Track monitoring Track induction training Replenish & re-profile ballast Lift and pack shovel/kango Tamping | 5 | 2 | 10 | Provision of competent Handback Engineer | 5 | 1 | 5 |
| 2 | Voids in ballast | Movement of track: Increasing wear Affecting clearances Causing cyclic top/uneven ride/twist faults Formation of wet beds Temporary speed restrictions Emergency speed restrictions De-railment | All | Void meters Track monitoring Track induction training Replenish & re-profile ballast Lift and pack shovel/kango Tamping | 5 | 2 | 10 | Provision of competent Handback Engineer | 5 | 1 | 5 |

| 3 | Missing clips, insulators and/or pads | Movement of track: Increasing wear Affecting clearances Causing cyclic top/uneven ride/twist faults Formation of wet beds Temporary speed restrictions Emergency speed restrictions De-railment | All | Track monitoring Track induction training Replace pads, insulators & clips Replenish & re-profile ballast Lift and pack shovel/kango Tamping | 5 | 2 | 10 | Provision of competent Handback Engineer | 5 | 1 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | Cracked welds/fish plates | Movement of track: Increasing wear Affecting clearances Causing cyclic top/uneven ride/twist faults Formation of wet beds Temporary speed restrictions Emergency speed restrictions De-railment | All | Track monitoring Track induction training Replace/clamp fish plates Install closure | 5 | 2 | 10 | Provision of competent Handback Engineer Impose Temporary speed restrictions or Emergency speed restrictions | 5 | 1 | 5 |
| 5 | Damaged sleeper or housing | Movement of track: Increasing wear Affecting clearances Causing cyclic top/uneven ride Formation of wet beds Temporary speed restrictions Emergency speed restrictions De-railment | All | Track monitoring Track induction training Replace sleepers/housings Fit tie bars | 5 | 2 | 10 | Provision of competent Handback Engineer Impose Temporary speed restrictions or Emergency speed restrictions | 5 | 1 | 5 |

| 6 | Incorrect gauge | Increasing wear De-railment | All | Track monitoring Track induction training Replace sleepers/housingsRe-rail/transpose rail Fit tie bars Adjust switch & re-profile | 5 | 2 | 10 | Provision of competent Handback Engineer Impose Temporary speed restrictions or Emergency speed restrictions | 5 | 1 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | Obstructions | Affecting operational running Affecting clearances De-railment | All | Track monitoring Track induction training Line blockage | 5 | 2 | 10 | Provision of competent Handback Engineer | 5 | 1 | 5 |
| 8 | Contaminated ballast Poor drainage | Erosion of ballast Movement of track: Increasing wear Affecting clearances Causing cyclic top/uneven ride Formation of wet beds Temporary speed restrictions Emergency speed restrictions De-railment | All | Cleaning or replacement of ballast Inspection & cleaning of drainage system Track monitoring Track induction training | 5 | 2 | 10 | Provision of competent Handback Engineer | 5 | 1 | 5 |
| 0 | Measuring equipment not calibrated | Incorrect readings leading to Cant errors Gauge errors Affecting clearances | All | Regular calibration of measuring equipment | 4 | 2 | 8 | Records of calibration retained by Handback Engineer and NCS | 4 | 1 | 4 |
| 10 | Incorrect reading or recording of measurements | Incorrect readings leading to Cant errors Gauge errors Affecting clearances | All | Literacy Numeracy Competency training in Handback Engineer duties | 4 | 2 | 8 | Records of Handback Engineer duties retained by NCS (use form NCS-FM-143.24) | 4 | 1 | 4 |

| 11 | Incorrect re-instatement of track | Movement of track: Increasing wear Affecting clearances Causing cyclic top/uneven ride Formation of wet beds Temporary speed restrictions Emergency speed restrictions De-railment | All | Literacy Numeracy Competency training & experience in Handback Engineer duties | 5 | 2 | 10 | Records of Handback Engineer duties retained by NCS (use form NCS-FM-143.24) Restore track in accordance with NR/SP/CTM/011 Competence & Training in Track Engineering | 5 | 1 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Matt Traynor

Managing Director

January 2021