| Location / Sites: Various | Task / Activity: Rail Site Management | # Trackside working near welding |
| --- | --- | --- |

| Name: Matt Traynor | Position: Managing Director | Date: Jan 2021 | Ref No: RSM06 |
| --- | --- | --- | --- |

| No | Hazard Identification | Associated risks | Who & Number at risk | Existing control measures | Consequence | likelihood | Current risk rating | Additional controls | Consequence | likelihood | Post risk rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Vehicles parked by lineside | Leakage or fuel and oil leading to possible poisoning of soil. Pollution of nearby watercourse | All | All vehicles used by NCS are leased and serviced in accordance with the manufacturers recommendations. | 3 | 2 | 6 | Petrol vehicles and plant all use unleaded petrol. Diesel vehicles are preferred to minimise fuel used NCS spillage procedure | 3 | 1 | 3 |
| 2 | Vehicles and plant left idling unnecessarily | Annoyance to neighbours and unnecessary fumes | All | All vehicles and plant to be shut down when not in use | 3 | 2 | 6 | . Generator and hydraulic packs are petrol powered, reducing noise and serviced to reduce fumes | 3 | 1 | 3 |
| 3 | Ultra violet light from electric arc welding | Annoyance to neighbours, unnecessary light pollution. Temporary, damage to eyes | All | Minimisation of adjacent personnel to process. | 3 | 2 | 6 | Use of lights shields in susceptible areas. Barriers in place when staff are vulnerable | 3 | 1 | 3 |
| 4 | Insufficient time to remove tools and equipment from the path of oncoming traffic | Tools and equipment struck by train | All | Provision of competent Safe Work Leader or COSS | 4 | 3 | 12 | Provision of competent Safe Work Leader or COSS - trained by approved training companies Competence assessments | 4 | 2 | 8 |
| 5 | Unable to provide sufficient warning time | Individual, materials and equipment struck by train | All | Provision of competent Safe Work Leader or COSS | 4 | 3 | 12 | Provision of competent Safe Work Leader or COSS - trained by approved training companies Lookouts competency assessed regularly | 4 | 2 | 8 |

| 6 | Work after dark or during inclement weather in red zone | Increased need for effective warning | All | Provision of competent Safe Work Leader or COSS | 4 | 2 | 10 | Provision of competent Safe Work Leader or COSS - trained by approved training companies Competence assessments | 4 | 1 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Matt Traynor

Managing Director

January 2021